| No. | Description |
| --- | --- |
|  | ROTARY MANUAL MICROTOME Micro and Coarse Advance movements in enclosed housing. Coarse advance left side Optimum specimen guidance by maintenance free cross roller guides. Safety lock on hand wheel. Section thickness adjustment from 0,5 µm to 60 µm in defined steps. Precise guide way and clamping device for knife holder with optimum sliding capability and prism guide. Increments : 0,5 µm from 0,5 µm to 2 µm 1 µm from 2 µm to 10 µm 2 µm from 10 µm to 20 µm 5 µm from 20 µm to 60 µm Vertical stroke: 70 mm Horizontal stroke: 40 mm Specimen Size Standard : maximum 55 mm x 55 mm With oil, dust cover and set of tools, Measurements: 460x 280x325mm. Supplied with following items - Knife Holder Basic Unit to accommodate different knife holders - 1No - Disposable Blade Holder Unit for Low /High Profile blades - 1No - Standard Object Clamp with fixed upper jaw ensuring maximum stability, maximum specimen size 50x50mm - 1No - Low/High Profile Disposable Blade (Pkt of 50Nos) - 1Pkt |